Barrowby Parish Council Email: clerk@barrowbyparishcouncil.gov.uk Address: Reading Room, Church Street, Barrowby, NG32 1BX Website: https://barrowby.parish.lincolnshire.gov.uk Grave Excavation Contractor Agreement Barrowby Burial Ground

This Agreement sets out the terms and conditions under which a contractor may undertake grave excavation and associated works within burial grounds operated by Barrowby Parish Council (“the Council”), acting as Burial Authority.

1. Parties

Burial Authority:
Barrowby Parish Council
The Reading Room
Church Street
Barrowby, Lincolnshire NG32 1BX

Contractor:
Name:

Company (if applicable):

Address:

2. Purpose of Agreement

The Contractor agrees to undertake grave excavation, preparation for interment, backfilling and reinstatement within the Council’s burial ground in accordance with:

- Burial Ground Regulations and Management Policies of the Council

- The Local Authorities’ Cemeteries Order 1977

- The Health and Safety at Work etc. Act 1974

- The Reporting of Injuries, Diseases and Dangerous Occurrences Regulations (RIDDOR)

- The Provision and Use of Work Equipment Regulations 1998 (PUWER)

- The Confined Spaces Regulations 1997

- ICCM Code of Safe Working Practice for Cemeteries

3. Contractor Status

The Contractor shall be deemed self-employed and independent and shall not be considered an employee of the Council.

The Contractor is responsible for:

- All staff, subcontractors and equipment used

- Payment of taxes and National Insurance

- Compliance with all applicable legislation

4. Scope of Works

The Contractor shall undertake the following services as instructed by the Council or the appointed funeral director:

- Excavation of graves for burial or interment

- Re-opening of existing graves

- Excavation of ashes plots where required

- Preparation of grave surrounds prior to the funeral

- Backfilling of graves after the service

- Reinstatement of turf and site condition

All works must be undertaken sensitively, respectfully and professionally.

5. Grave Dimensions and Depth

All graves must comply with the minimum requirements of the Local Authorities’ Cemeteries Order 1977, including:

- Coffins must be separated by at least 150mm (6 inches) of earth when reopening graves.

- No part of a coffin shall be less than 900mm (3 feet) below ground level.

Typical minimum depths:

| Burial Type | Minimum Depth |
| --- | --- |
| Single burial | approx. 4’6” |
| Double burial | approx. 6’0” |

The Council may specify alternative depths depending on ground conditions.

6. Health and Safety

The Contractor shall:

- Comply with all Health and Safety legislation

- Provide risk assessments and method statements

- Ensure all staff are competent and trained

- Ensure safe working around excavations

- Use appropriate shoring equipment where required

- Ensure graves are securely covered or protected when unattended

A ladder must remain in place when working within a grave excavation and appropriate PPE must be worn.

7. Insurance

The Contractor must hold the following minimum insurance:

- Public Liability Insurance – £5,000,000 minimum cover

- Employer’s Liability Insurance (if staff employed)

Copies of valid insurance certificates must be provided annually.

The Contractor shall indemnify the Council against any claims arising from their work.

8. Equipment

The Contractor shall supply all equipment necessary for grave excavation including:

- Excavation machinery (where appropriate)

- Shoring equipment

- Boards, staging and grass matting

- Tools required for hand excavation

- Protective barriers

All equipment must comply with PUWER regulations.

9. Timing of Works

Unless otherwise authorised:

- Graves must not be excavated more than 48 hours before the interment

- All graves must be properly protected when unattended.

The grave must be ready in advance of the funeral service.

10. Protection of Surrounding Graves

The Contractor shall ensure:

- Adjacent memorials and graves are protected

- Turf and soil are stored appropriately

- Vehicles remain on designated routes

- Any damage is repaired immediately at the Contractor’s expense.

11. Spoil and Waste

Excavated spoil must:

- Be placed in the designated location

- Be removed from site where required

- Not obstruct pathways or adjacent graves

12. Backfilling and Reinstatement

Following the funeral service the Contractor shall:

- Backfill the grave safely

- Compact soil appropriately

- Reinstate turf

- Leave the site clean and safe

- Place floral tributes on the grave where appropriate

13. Incident Reporting

The Contractor must immediately report to the Council:

- Any accident or dangerous occurrence

- Any damage to memorials or graves

- Any ground instability or safety concern

Incidents must be reported in accordance with RIDDOR.

14. Compliance with Burial Ground Regulations

The Contractor agrees to comply with:

- All Burial Ground Regulations

- Directions issued by the Parish Clerk or authorised officer

- Any additional instructions issued on site.

Failure to comply may result in suspension from working within the burial ground.

15. Termination

The Council may terminate this agreement immediately where:

- Health and safety requirements are breached

- Insurance lapses

- Work is undertaken negligently or unsafely

- Burial ground regulations are not followed.

16. Fees

Fees for grave excavation shall be agreed between the Council and the Contractor and may be reviewed annually.

17. Duration

This agreement shall remain valid for:

☐ One year
☐ Three years

Unless terminated by either party with 30 days written notice.

18. Signatures

For Barrowby Parish Council

Name: ________________________

Position: Clerk / Authorised Officer

Signature: _____________________

Date: _________________________

Contractor

Name: ________________________

Signature: _____________________

Date: _________________________

Document control

- Owner: Burial Ground Committee

- Responsible officer: Clerk / Proper Officer

- Adopted: [date] (Minute ref: [ ])

- Version:  2026.1

- Review: Annually or earlier if legislation/guidance changes

- Next review due: January-March 2027 at Burial Ground Committee meeting for formal adoption at Parish Council May 2027